The Motorola -- MaxTrac�LS LTR Trunking -- Radio Service Software,

RVN-4139, has been updated as follows;

Part #......................... RVN-4139-A

includes 3.5" installation diskette

RSS User's Manual #....... 68P8-1078C79

Software Version............... R01.00.00

Software Date.................. 06-MAY-95

1. - The SITE Sub-Directory is now the DEFAULT storage for SITE FILES.

2. - !! IMPORTANT !!  The STACK size of this RSS has been increased,

to enhance stability in a normal DOS environment (this does NOT affect

execution memory requirements).  If your environment includes elements

which create abnormal quantities of hardware/software interrupts, then

we STRONGLY recommend the following statement be inserted into your

CONFIG.SYS file (DOS 5.x/6.x):

STACKS=8,512

This tells DOS to furnish stack space to hardware interrupts which

occur during RSS execution, thereby reducing the RSS stack loading, and

avoiding errant DOS/application behaviours.  This statement WILL reduce

the amount of conventional memory available by approximately 4 Kb.

3. - The RSS CodePlug ARCHIVE and SITE File -BACKUP- default is now

[path] Sub-Directory        [C:\MRSS\MAXLS\] BACKUP.

For FLOPPY diskette backup, Please see Item 4-(B) below.

4. - The Install batch file, A:HDINSTAL.BAT;

(A) Defaults installation to Hard Drive C:\MRSS\MAXLS; you can

edit HDINSTAL.BAT to change default drive and/or the sub-directory.

(B) Creates sub-directory "C:\MRSS\MAXLS\BACKUP".  To backup

on floppy diskettes, use MAIN MENU "F9 - SETUP" and "F3 - PC CONFIG"

and enter "drive_letter:" (e.g. "A:") in the Backup field.  If

you wish this to be the default setup, use "F8 - SAVE" to save the

Configuration File.  Also, temporary setting can be done via the

"F2 - CHANGE ARCHIVE" function on LOAD/SAVE sub-menus.

5. - If you are replacing RSS HVN-9645 (Radius/M500 LTR Trunking)

with this RSS, Please Read Appendix (-A-), below.

6. - The INSTALLATION DISKETTE for this RSS has all files marked "Read Only"

via the DOS command "attrib +r *.*".  It is recommended that the user

never execute the RSS from this original diskette.  If a DOS "diskcopy"

of the original is made, the attribute bits are also copied.  On the

COPY, the following files should be unlocked via the DOS command

"attrib -r <file>", before execution;

MAXT_LS.CFG

7. - This RSS executes under DOS, in conventional memory (1st 640 KB memory).

After INITIAL execution, conventional memory is dynamically allocated

for data/file functions ( e.g. more files = more memory required ),

and if the RSS cannot allocate enough conventional memory, an error

message is given. Make more conventional memory available by using DOS

"HIMEM.SYS" and "DOS=HIGH", and using DOS 5x/6x "MEM /C" to list memory

allocations from CONFIG.SYS and AUTOEXEC.BAT, removing any unnecessary

items from those files.

8. - Corrected RSS handling of CTRL-BRK and CTRL-C; Execution of RSS is no

longer stopped by these keystrokes.

9. - Communications Port (COM1 - COM2) IC (UART) now completely

initialized upon startup.  Prevents interference from prior

applications when communicating with Radio via the RIB.

10. - Update to interface more reliably with the Radio on faster 386/486

machines.  This version has Radio Bus Timing controlled by PC Timer IC.

11. - This RSS now has no EXTERNAL OVERLAY (*.OVL) files.  This allows

slightly faster loading and execution, and reduces execution memory

requirements.  The RSS executable file (*.EXE) MUST be run from the

working directory/disk, so that INTERNAL OVERLAYS can be found.

To minimize hard disk space usage, use DOS command "DEL *.OVL" to delete

the old overlay files in the installation Directory.

12. - Warning message text now contained in new file ERRORS.DAT.

13. - Corrected problems with HELP screen printing; 1st line of

successive screens (after 1st) no longer blanked.

NOTE: This RSS is NO LONGER PACKAGED on 5.25" floppy diskettes.

(C) Copyright MOTOROLA Inc. 1986, 1995.  All Rights Reserved.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Appendix (-A-);  -  Converting From RSS HVN-9645 (Radius/M500 LTR Trunking)

You are strongly advised to perform a complete PC-DOS backup (via DOS "COPY"

or "XCOPY" command) of ALL files in the following sub-directories, to a

floppy diskette, BEFORE performing the below installation;

(Drive:)\MRSS\RADMTRNK

(Drive:)\MRSS\RADMTRNK\ARCHIVE

The major differences between HVN-9645 and this RSS package are the default

PATH and FILE names under the PC-DOS Operating System.  This RSS creates the

following DEFAULT paths for normal installation:

(Drive:)\MRSS\MAXLS         -   Main Executable/Utility Files

(Drive:)\MRSS\MAXLS\ARCHIVE -   CODEPLUG and ARCHIVE DATABASE Files.

(Drive:)\MRSS\MAXLS\SITE    -   SITE and ARCHIVE DATABASE Files.

(Drive:)\MRSS\MAXLS\BACKUP  -   Default Backup Sub-Directory For -Both-

CODEPLUG and SITE Files/DataBases.

Normal installation will create all four sub-directories, and copy the Main

Executable and Utility files from the Installation diskette.  You will need

to copy your existing CODEPLUG, SITE and ARCHIVE DATABASE files to the new

sub-directories, and RE-NAME the ARCHIVE DATABASE FILES.

STEP                        ACTION / DOS COMMANDS

----                        ---------------------

1) Install this RSS;        Insert Diskette in Floppy Drive (A: or B:).

Type "(A: or B:)", Then "HDINSTAL (C: or D:)".

2) Go To CODEPLUG Archive;  Type "(C: or D:)", Then "CD \MRSS\MAXLS\ARCHIVE".

3) Copy CODEPLUG FILES;     "COPY  \MRSS\RADMTRNK\ARCHIVE\A*.*".

"COPY  \MRSS\RADMTRNK\ARCHIVE\B*.*".

4) Copy ARCHIVE DATABASE    "COPY  \MRSS\RADMTRNK\ARCHIVE\RADMTRNK.DBF".

5) ReName ARCHIVE DATABASE  "REN  *.DBF  MAXT_LS.DBF".

6) Go To SITE FILE Archive  "CD  ..\SITE".

7) Copy SITE FILES;         "COPY  \MRSS\RADMTRNK\ARCHIVE\SITE*.*"

8) Copy ARCHIVE DATABASE    "COPY  \MRSS\RADMTRNK\ARCHIVE\RADMSITE.DBF".

9) ReName ARCHIVE DATABASE  "REN  *.DBF  MAXT_LSI.DBF".

At this point, if you wish to restore backup codeplug/site/database files

from a floppy diskette, you can;

10) Go To BACKUP Sub-Dir.;  "CD  ..\BACKUP".

11) Copy backup files;      "COPY  (A: or B:)*.*".