MOTOROLA, INC.

COPYRIGHT 1989, 1990, 1991, 1992

SPECTRA RADIO SERVICE SOFTWARE UPDATE

-------------------------------------

THIS README.DOC FILE IS FOR VERSION R05.03.00

RVN4001K

SOFTWARE ERRORS CORRECTED/ENHANCEMENTS SINCE R05.01.00 RELEASE

--------------------------------------------------------------

1)  This version of RSS is necessary to correctly install certain

firmware enhancements. Included in this version is a file

named patch.dat. This file contains firmware enhancements

which correct improper mode display on mode changes and

a problem with direct mode entry config. Also included is a

correction for a timing error with securenet which could

have caused the radio to lock up under certain conditions.

2)  This RSS corrects an error whereby talkgroups could be

corrupted on archives where the trunked modes were not

configured contiguously with their respective personalities.

3)  This release correctly unpacks more than 1 channel number

block for UHF/VHF trunked units.

4)  Direct Zone buttons are correctly programmed on the DEKS

of units with the new enhanced or rotary control heads.

5)  Required codeplug space is greatly reduced with some models.

As a result of this change, Once this version of RSS is used

older versions are incompatible. The older versions will detect

this change and there is no danger of corruption.

6)  This version of RSS, along with 6.03 version of firmware is

required to use the horn ring option.

SOFTWARE ERRORS CORRECTED/ENHANCEMENTS SINCE B05.02.00 RELEASE

--------------------------------------------------------------

1)  This version of RSS correctly allows the user to select

steered modes for VRS on models with the SIU control head.

2)  Corrected an error with overlapping fields for Metrocom radios.

Previously, the domestic enable field and location receiver

field occupied the same location for 800 Mhz radios.

3)  This software insures that when a trunked mode is copied

from another mode on a zone/mode radio that the new mode

is put in the copied from mode's zone.

REINSTALLING SPECTRA RSS?

------------------------

If  you  are reinstalling Spectra Radio Service  Software  for

this update (or for any reason), you should remember to  check

the setup configuration.

When  'INSTALL.EXE' is run, only the Radio  Service  Software

program  files  will  be over written and  not  the  setup  or

archive  files ('SPECTRA.CFG' and 'SPECTRAM.DBF)'.  This  will

effectively  update the software and leave all  archive  paths

and files intact.

NOTE : THIS VERSION OF RSS WILL NOT EXECUTE ON FLOPPIES, IT

MUST BE INSTALLED TO A HARD DISK.

PROBLEMS WITH RUNNING SPECTRA RSS?

---------------------------------

In  general, some memory resident programs such as a RAM  disk

or  a print spooler may interfere with the  memory  management

routines used by the Radio Service Software.  If you encounter

a  problem  you  should first try to  run  the  Radio  Service

Software  after 'booting' the computer off of a  standard  DOS

disk  which  has  no other 'AUTOEXEC.BAT'  files.   Then,  you

should  begin  to  systematically  load  the  memory  resident

programs  followed  by the Radio Service  Software  until  the

error condition is encountered again. Once the incompatibility

is  found, it may be possible to change the loading  sequence,

program parameters, etc. in order to alleviate the problem.

The  Spectra Radio Service Software is not designed to run  on

Local  Area Network (LAN) systems.  In most cases it will run,

but multiple  users may  not be able to access  archive  files

simultaneously.