2. SPECIFICATIONS

GENERAL SPECIFICATIONS

RADIO INTERFACE SPECIFICATIONS

ADDITIONAL SPECIFICATIONS

PROGRAMMABLE FUNCTIONS

General

User Specific

Installation Aids

## GENERAL SPECIFICATIONS

Power		11 to 16 volts DC, 100 mA nominal

Operating Temperature	0° to 60° Celsius

Size		5.5 in W ´ 6.25 in D ´ 1.4 in H

Weight		1.0 pound

## RADIO INTERFACE SPECIFICATIONS

Connections	12 volt DC, ground, RX Discriminator Audio, TX Mic Audio, TX PTT, RX Carrier Detect, CTCSS/DCS Encode

PTT		FET pull to ground

Carrier Detector	External COR input with polarity and threshold adjustments

TX Audio	-40 to +6 dBm, Hi/Lo range selector, 1 kW output impedance

RX Audio	-40 to +10 dBm, Hi/Lo range selector, 25 kW input impedance

CTCSS/DCS encode	-40 to +3 dBm, Hi/Lo range selector, 600 W output impedance

CTCSS slope	Flat or de-emphasized output

## ADDITIONAL SPECIFICATIONS

Indicators	Status, Carrier, Transmit, Power

Station ID	Morse Code, fixed 1200 Hz tone frequency, and programmable call sign

Programming	Via DTMF radio

Data Retention	Nonvolatile EEPROM

## PROGRAMMABLE FUNCTIONS

### General

Mode		Carrier repeat, or validated CTCSS/DCS users

Station ID	ID interval from 1 to 90 minutes, call sign programmable up to 8 characters, periodic ID or ID based on transmit activity

TX Time-out	1 to 9 minutes in 1-minute steps

TX Hold Time	0 to 9 seconds in 1-second steps

Alternate TX Hold Time	0 to 9 seconds in 1-second steps

Courtesy Tone Frequency	500 Hz, 1000 Hz, 1500 Hz selectable

### User Specific

Number of Users	155 database entries. User numbers are from a cross-reference table based on CTCSS/DCS decode, 0 = Carrier repeat, 1 to 50 = CTCSS, 51 to 154 = DCS. Commands are available to program a single user, or globally for all users.

User Status	Disabled, enabled, or reserved. Reserved users will actuate the transmitter and generate the proper CTCSS/DCS for dispatch, but will not pass repeat audio. Valuable for no-pay customers.

Regenerated CTCSS/DCS	Selectable encode may be any CTCSS or DCS, programmable per user

TX Hold Time	Normal or Alternate transmit hold time selection

Tone-in-Tail	Enable or disable CTCSS/DCS encode during TX hold time

Courtesy Tone	Enable or disable

### Installation Aids

DCS Polarity	Programmable data inverters for encode and decode

Program Mode Access	Five-digit DTMF password selectable

Clear		Resets unit to all factory default settings

TX Audio Test	Generates 1 kHz audio tone for transmit deviation adjustment

Repeat Audio Test	Enables carrier repeat operation to adjust RX audio for unity repeat gain

CTCSS Level Test	Keys transmitter with CTCSS tone to check encode level

CTCSS Pre-emphasis Test	Verifies that CTCSS encode frequency response is flat